Before.

After Removed.

Step 1: Preparing the Truck

Remove the factory adhesive from the top ledge of the window cavity using a 90 degree sander.

Painted over to prevent rusting and to finish off what areas may still be visible when the window is opened. Use the black Rust-Oleum enamel paint and a small foam brush. Apply one even coat.

After paint.

The Rust-Oleum paint requires 24 hours to dry.

Step 2: Drilling the Mounting Holes

Use two people to properly place this pattern.

Place the pattern tight to the window cavity. With another individual holding the pattern steady in this position, drill through the bushings on the pattern with a 7/32” drill bit.

Step 3: Installing Egress   Window

Holding the window at an angle will help expose a workable amount of thread.

With one person on the interior of the vehicle, carefully lift the window into place. Place a #8 washer and 8mm nut on each of these bolts and completely tighten two them.  Shut the window and test for appropriate fit. Fasten the remaining bolts and install the new weather stripping.

Required Tools

Drill with 7/32” Drill Bit

Foam Paint Brush

Black Rust-Oleum Paint

Window Pattern

Five 8mm Nuts

Five #8 Washer

8mm Socket Wrench

Angle Sander

Bottom of window to chassis.

Window to window.

Check these two seams for proper spacing. Also, confirm that the window will open and close properly.

Step 4: Water Test

With an individual on the interior of the vehicle to spot for leaks, spray water at the window from all directions. Check the seam on the interior where the weather stripping meets the valance.